## REQUEST FOR FINAL FABRICATION

- BOARD SIZE: 3.00” X 3.00”.   0.008” MIN COPPER THICKNESS, 4 LAYER BOARD.

LIST OF GBR FILES:

- 423B1-1.PHO & .REP			LAYER 1 - TOP LAYER

- 423B1-2.PHO & .REP			LAYER 2 – GND PLANE

- 423B1-3.PHO & .REP			LAYER 3 – GND PLANE

- 423B1-4.PHO & .REP			LAYER 4 - BOTTOM LAYER

- 423B1-TS.PHO & .REP			TOP SILKSCREEN

- 423B1-BS.PHO & .REP			BOTTOM SILKSCREEN

- 423B1-TM.PHO & .REP			TOP SOLDER MASK

- 423B1-BM.PHO & .REP			BOTTOM SOLDER MASK

- 423B1-TP.PHO & .REP			TOP SOLDER PASTE

- 423B1FAB.PHO & .REP			DRILL DRAWING LAYER (FAB DWG)

- 423B1DRL01.DRL			DRILL FILES

- 423B1DRL01.LST			DRILL FILES

- 423B1DRL01.REP			DRILL FILES

- PANELIZE 20 BOARDS ON A 14” X 17” PANEL (5 X 4 ARRAYS),  .020” AIRGAP BETWEEN BOARDS.  TOTAL OF 440 BOARDS ON 22 PANELS.

- REQUIRE TO GET 20 BDS FOR THE 1ST ARTICLE AND THE REST WILL BE DONE UPON APPROVAL.

- MATERIAL: FR4, .062” THICKNESS, 2 OZ CU, TIN-LEAD REFLOW FINISH.  FILL VIAS WITH SOLDERS.

- PLEASE LOOK AT THE FABRICATION DRAWING FOR THE OTHER REQUIREMENTS FOR THE JOB.

- PLEASE CONTACT KIM TRAN AT  408-954-8400 X 3411 IF YOU HAVE ANY QUESTION.

THANK YOU

KIM