# DC385 Introduction

## Description

Demonstration circuit DC385 is a high density synchronous step-down (buck) regulator using the LTC1778 No RSENSE™ switching regulator controller. This controller uses a valley current control architecture to deliver very low duty cycles without requiring a sense resistor. It provides high efficiency operation at light loads by means of discontinuous mode operation. Noise and RF interference are reduced by means of a forced continuous control pin. The circuit uses a dual SyncFET™ and has a very small footprint. With a wide operating range of VIN from 5V to 28V and a duty cycle of 2% to 90% at 200kHz, the circuit demonstrates that the LTC1778 is ideal for applications such as notebook and palmtop computers, PDAs, battery chargers and distributed power systems.

## Quick Start Guide

This demonstration board is easily set up to evaluate the performance of the LTC1778. Please follow the procedure outlined below for proper operation.

- Connect input power to the VIN and GND terminals. Input voltage is limited to between 5V to 28V. Refer to Figure 1 for proper measurement equipment setup.

Connect the load between the VOUT and GND terminals.

Figure 1. DC385 Test and Measurement Setup

- Connect the FCB signal to the SGND terminal to force continuous synchronous operation at low loads; connect the FCB signal to the INTVCC signal to enable discontinuous mode operation at low loads (see solder pads on bottom side of assembly).

- To shut down the circuit, connect the RUN/SS signal to the SGND terminal. Disconnect the RUN/SS signal from the SGND terminal to enable normal operation.

When measuring input or output ripple, see Figure 2 for proper scope probe technique.

Figure 2. Scope Probe Position for Ripple Measurement

SyncFET is a trademark of International Rectifier