## DC503A DEMO BOARD QUICK START GUIDE

Description:

The DC503A demo circuit board is intended to demonstrate the capabilities of the LT5512 downconverting mixer IC for cable infrastructure applications.

The LT5512 (Figure 1) is a broadband high signal level mixer IC optimized for high linearity downconverter applications including cable and wireless infrastructure.  The IC includes a differential LO buffer driving a double-balanced mixer.  An integrated RF amplifier improves LO-RF isolation and eliminates the need for precision external bias resistors.

The LT5512 is a high-linearity alternative to passive diode mixers.  Unlike passive mixers, which have conversion loss and require high LO drive level, the LT5512 delivers conversion gain and requires significantly lower LO drive levels.

The DC503A demo board downconverts 1230MHz RF input to 270-870MHz IF output.  The LO is injected from the high-side between 1500MHz and 2100MHz.

Figure 1. LT5512 IC Block Diagram

LT5512 Downconverting Mixer Single-Tone Measurements for 1230 MHz CATV Band.

- Connect all test equipment as shown in Figure 2.

- Set the Power Supply output voltage to 5V, and set the current limit to 100mA.

- Connect TP1 (EN) to the 5V power supply.  Mixer is in ENABLE mode.

- Set the Signal Generator #1 to provide an 1800MHz, -10dBm, CW signal to the demo board LOin port.

- Set the Signal Generator #2 to provide a 1230MHz, -10dBm, CW signal to the demo board RFin port.

- Set the Spectrum Analyzer’s center frequency to 570MHz.  Perform mixer conversion gain, and 1dB compression measurements.

- Set the Spectrum Analyzer’s center frequency to 1800MHz.  Perform mixer LO to IF leakage measurement.

SIGNAL

GENERATOR 1

SIGNAL

GENERATOR 2

SPECTRUM

ANALYZER

POWER SUPPLY

Figure 2. Test Set-Up for Mixer Single-Tone Measurements.

LT5512 Downconverting Mixer Two-Tone Measurements for 1230MHz CATV Band.

- Connect all test equipment as shown in Figure 3.

Note:  1). A high quality combiner that provides 50-ohm termination on all ports and has good port-to-port isolation should be used.

2). The two 3dB attenuators on the inputs of the combiner are used to further improve source isolation.

- Set the Power Supply output voltage to 5V, and set the current limit to 100mA.

- Connect TP1 (EN) to the 5V power supply.  Mixer is in ENABLE mode.

- Set the Signal Generator #1 to provide a 1800MHz, -10dBm, CW signal to the demo board LOin port.

- Set the Signal Generator #2 and #3 to provide two -10dBm CW signals to the demo board RFin port—one at 1229.9MHz, and the other at 1230.1MHz.

- Set the Spectrum Analyzer’s center frequency to 570MHz.  Perform input 3rd order distortion measurement.  (IIP3 = P1 + (P1 – P3) / 2 – Gc, where P1 is the lowest power level of the two desired output tones at either 569.9MHz or 570.1MHz, P3 is the largest 3rd order product at either 569.7MHz or 570.3MHz, and Gc is conversion gain.  P1 and P3 are in units of dBm, and Gc is in dB.)

SIGNAL

GENERATOR 1

SIGNAL

GENERATOR 2

SPECTRUM

ANALYZER



3dB PAD

3dB PAD

SIGNAL

GENERATOR 3

POWER SUPPLY

Figure 3. Test Set-Up for Mixer Two-Tone Measurements.

- APPROVED VENDOR LIST

| VENDOR | PHONE NUMBER | PART TYPE | WEBSITE ADDRESS |
| --- | --- | --- | --- |
| AAC (was TAD) | (800) 508-1521 | CHIP RESISTORS |  |
| AAC (was TAD) | (714) 255-9186 |  |  |
| AAVID | (714) 556-2665 | HEAT SINKS |  |
| ALLEN BRADLEY | (800) 592-4888 | CARBON RESISTORS |  |
| AMP | (717) 564-0100 | PC MOUNT BNC |  |
| APEM | (718) 246-1007 | SMD TOGGLE/PB SWITCH |  |
| API DELEVAN | (716) 652-3600 | INDUCTORS |  |
| AVX | (843) 946-0362 | CHIP CAPS |  |
| AVX | (843) 946-0524 | CHIP RESISTORS |  |
| AVX | (207) 282-5111 | TANTALUM CAPS |  |
| AVX | (843) 946-0323 | HIGH VOLTAGE CAPS |  |
| BERG | (800) 237-2374 | CONNECTORS |  |
| BH ELECTRONICS | (952) 894-9590 | INDUCTORS |  |
| BI TECHNOLOGIES | (714) 447-2656 | TRANSFORMERS |  |
| BI TECHNOLOGIES | (714) 447-2345 | RES./RES. NETWORKS |  |
| BOURNS | (801) 750-7253 | POTENTIOMETERS, SIPS |  |
| CADDOCK ELECTRONICS | (541) 496-0700 | HIPO. RES., SIPS, DIPS |  |
| CENTRAL SEMI | (631) 435-1110 | SMALL SIGNAL DISCRETES |  |
| CHICAGO MINIAT. LAMP | (201) 489-8989 | LEDS |  |
| COILCRAFT | (847) 639-6400 | INDUCTORS |  |
| COMM CON | (626) 301-4200 | HEADERS, SHUNTS |  |
| CONNEX | (805) 378-6464 | BNC CONNECTORS |  |
| COOPER ELECT. TECH. | (561) 752-5000 | INDUCTORS |  |
| CORNELL DUBILIER | (508) 996-8561 | CAPACITORS |  |
| CTS | (219) 293-7511 | RESISTOR ARRAYS |  |
| CUI-STACK | (503) 643-4899 | POWER CONNECTORS |  |
| DALE (see Vishay) | (605) 665-1627 | INDUCTORS |  |
| DALE (see Vishay) | (605) 665-9301 | SENSE RESISTORS |  |
| DATA DISPLAY PRODUCT | (800) 421-6815 | LEDS |  |
| DIODES INC. | (805) 446-4800 | DIODES |  |
| ELECTRONIC CONCEPTS | (908) 542-7880 | 400V FILM CAPACITORS |  |
| EPSON | (310) 787-6300 | CRYSTALS |  |
| FAIRCHILD | (207) 775-4502 | LOGIC |  |
| FAIRCHILD | (408) 822-2126 | MOSFETS |  |
| FAIRCHILD | (888) 522-5372 | CRG (CUST. RESPONSE) |  |
| FCI | (717) 767-8005 | HOT PLUG CONNECTORS |  |
| FUKUSHIMA | (818) 765-8300 | MPC RESISTORS |  |
| FUJI | (201) 712-0555 | SCHOTTKY DIODES |  |
| GENERAL SEMICONDUCTOR | (516) 847-3000 | DIODES |  |
| GOWANDA | (716) 532-2234 | INDUCTORS |  |
| GRAYHILL | (708) 354-1040 | DIP SWITCHES |  |
| HARRIS | (800) 442-7747 | LOGIC |  |
| HEWLETT PACKARD | (800) 235-0312 | IR LED |  |
| HITACHI | (408) 433-1990 | RF POWER AMPS |  |
| IDT | (408) 727-6116 | LOGIC IC |  |
| IR | (310) 322-3331 | MOSFETS |  |
| IRC | (361) 992-7900 | SENSE RESISTORS |  |
| ITW PAKTRON | (708) 667-3444 | CAPACITORS |  |
| JOHNSON COMPONENTS | (650) 948-6533 | RF CONNECTORS |  |
| JOHNSON COMPONENTS | (760) 434-5961 | RF CONNECTORS |  |
| KEMET | (408) 986-0424 | TANTALUM CAPS |  |
| KEMET | (864) 963-6300 | CRG (CUST. RESPONSE) |  |
| KETEMA | (714) 630-0081 | SURGE SUPPRESSORS |  |
| KEYSTONE | (718) 956-8900 | JACKS, TURRETS |  |
| LITEON | (408) 241-4588 | LEDS, DIODES |  |
| LTC | (408) 432-1900 | HIGH PERF. I.C.S |  |
| MAGNETICS | (800) 245-3984 | TOROID CORES ETC. |  |
| MARCON | (847) 696-2000 | HIGH C/V CAPACITORS |  |
| METHODE | (800) 323-6864 | ZIF SOCKETS |  |
| MF ELECTRONICS | (914) 576-6570 | CRYSTAL OSCILLATORS |  |
| MICROCHIP | (602) 786-7200 | MICROCONTROLLER IC |  |
| MICRO PLASTICS | (870) 453-8861 | NYLON STANDOFFS |  |
| MICRO-SEMI | (617) 926-0404 | DIODES |  |
| MIDCOM | (605) 886-4385 | INDUCTORS |  |
| MIDCOM | (800) 643-2661 | INDUCTORS |  |
| MILL-MAX | (516) 922-6000 | TURRETS |  |
| MINICIRCUITS | (718) 934-4500 | RF TRANSFORMERS |  |
| MOTOROLA | (800) 441-2447 | LOGIC, REGS |  |
| MURATA ELECTRONICS | (770) 436-1300 | CAPS., INDUCTORS, |  |
| MURATA ELECTRONICS | (800) 831-9172 | CRG (CUST. RESPONSE) |  |
| MURATA ELECTRONICS | (770) 433-5789 | RF DEVICES |  |
| NEC/TOKIN | (510) 324-4110 | INDUCTORS/HI C/V CAPS |  |
| NICHIA | (408) 573-0933 | WHITE LEDS |  |
| NICHICON | (847) 843-7500 | ELECTROLYTIC CAPACITOR |  |
| ON SEMICONDUCTOR | (602) 244-6600 | DISCRETE DIODES ETC. |  |
| ON SHORE | (602) 921-3000 | TERMINATORS |  |
| PANASONIC | (714) 373-7334 | INDUCTORS, POLY CAPS |  |
| PANASONIC | (201) 348-5217 | LEDS |  |
| PANASONIC | (201) 373-7334 | SWITCHES |  |
| PERICOM | (408) 435-0800 | LOGIC IC |  |
| PHILIPS | (914) 246-2811 | INDUCTORS |  |
| PHILIPS | (914) 247-2036 | PLANAR INDUCTORS |  |
| PHILIPS | (508) 851-2200 | DISCRETES, I.C.s |  |
| PULSE | (619) 674-8100 | INDUCTORS |  |
| QT OPTOELECTRONICS | (408) 720-1440 | RF SWITCH |  |
| RAYCHEM | (800) 227-4856 | FUSES |  |
| RG ALLEN | (818) 765-8300 | METAL OXIDE RESISTORS |  |
| RF MICRO DEVICES | (336) 664-1233 | RF2138 / RF2140 |  |
| SAMTEC | (800) 726-8329 | WIRE JUMPERS |  |
| SANYO | (619) 661-6835 | OSCON CAPS |  |
| SCHOTT | (507) 532-3201 | INDUCTORS, XFORMERS |  |
| SCHURTER | (707) 778-6311 | FUSES AND HOLDERS |  |
| SIGNATRON | (909) 464-1883 | DB9 CONNECTORS |  |
| SIEMENS | (108) 257-7910 | OPTO |  |
| SILICONIX | (800) 554-5565 | MOSFETS |  |
| SILICONIX | (408) 988-8000 | MOSFETS |  |
| SPRAGUE | (207) 324-4140 | CAPACITORS |  |
| SULLINS | (760) 744-0125 | HEADERS, SHUNTS |  |
| SUMIDA | (847) 956-0667 | INDUCTORS |  |
| SUMIDA | (408) 982-9660 | INDUCTORS |  |
| TAIYO YUDEN | (408) 573-4150 | CHIP CAPS / RES. |  |
| TAIYO YUDEN | (800) 348-2496 | CRG (CUST. RESPONSE) |  |
| TEKTRONIX | (800) 835-9433 | SCOPE PROBE SOCKETS |  |
| TEMIC | (408) 970-5700 | IR PHOTO DIODE |  |
| THERMALLOY | (972) 243-4321 | HEAT SINKS |  |
| THIN FILM TECHNOLOGY | (507) 625-8445 | THIN FILM CHIP RESISTORS |  |
| TOCOS | (847) 884-6664 | SMD POTENTIOMETERS |  |
| TOKIN (NEC) | (510) 324-4110 | CAPS., INDUCTORS, |  |
| TOKO | (847) 699-3430 | RF PRODUCTS |  |
| TOSHIBA | (714) 455-2000 | SINGLE GATE LOGIC |  |
| TOSHIBA | (949) 455-2000 | LOGIC |  |
| UNITED CHEMICON | (847) 696-2000 | ELECTROLYTIC CAPACITOR |  |
| VISHAY | (605) 665-9301 | ZENER/SM. SIGNAL DIODES |  |
| VISHAY | (605) 665-9301 | INDUCTORS, SENSE Rs |  |
| VITRAMON | (203) 268-6261 | CERAMIC CHIP CAPACITOR |  |
| WIMA | (914) 347-2474 | PAPER/FILM CAPACITORS |  |
| ZETEX | (631) 366-5068 | SMALL SIGNAL DISCRETES |  |
| ZIERICK | (800) 882-8020 | STAKED PINS |  |